Room CO2,  Temp & Humidity
Monitoring

CO2, Temperature & Humidity Monitor

PART 1 – GENERAL

- RELATED DOCUMENTS

  - Drawings and general provisions of the Contract, including General and Supplementary General Conditions and other Division 01 Specification Sections, apply to this Section

PART 2 – PRODUCTS

  - Wall Mount CO2, Temperature & Humidity Monitor (S&S Middle East RSCO2TH Range)

    - Product supplied shall be a wall mount combined carbon dioxide (CO2), temperature & humidity monitor rated for indoor environments.

    - Monitor shall be available with Bacnet MS/TP or Modbus RTU communication protocols options.

    - Monitor shall have internal volt free relays & 0-10V output to activate ventilation equipment and warning alarms upon reaching alarm thresholds.

    - Monitor shall be available with blank, LED or display screen options.

    - Monitor shall include a combination of CO2, temperature & humidity.

    - Monitor range shall be available as 24VAC/DC or 230VAC powered.

    - Monitor shall have an operating temperature range: -10°C to 50°C.

    - Temperature sensor accuracy shall be: ±0.5° @ 10-45°C.

    - Humidity sensor accuracy shall be: ±2% @ 15-30°C (20-90%RH)

    - System shall be 5.95” x 4.37” x 1.87”

    - Acceptable product range: S&S Middle East RSCO2TH

PART 3 – EXECUTION

  - INSTALLATION

    - Sensors must be installed in compliance with the manufacturer's guidelines.

    - Install wiring and conduit to prevent interference with other systems.

    - Ensure accessibility for future sensor maintenance and calibration.